Sessional Examination-2022

Paper Name: Digital Systems and Applications

Code: PHY-HC-3036

Total Marks: 30

- Answer all the question							 (9 x 1=9)

- Define active components and passive components.

- Each term in the standard POS form is called a ……….. (fill in the blanks)

- Which of the following gate cannot be used as an inverter

  - NOR      b. NAND       c. X-NOR        d. OR

- Write down one advantage and one disadvantage of using ICs over discrete component circuits.

- A multiplexer is a

- 1-to-N device 	b. N-to-1 device   	c. 1-to-1 device  d. N-to-N device

- The output of a logic gate is HIGH when atleast one of its inputs is LOW. It is true for

  - X-OR     	b. NOR   	c. NAND         d. OR

- Why AND and OR gate are not called universal gate?

- What is BCD?

- The logic expression AB+AB  can be implemented by giving the inputs A and B to a two-input

- X-OR     	b. X-NOR   	c. NAND         d. NOR

- Answer any six questions                                                                      (6 x 2=12)

- How can an X-OR and an X-NOR gate be used as an inverter individually?

- Draw the logic diagram using only two input NAND gates to implement the following expression Y= (AB+AB)

- Define minterm and maxterm.

- What are linear and digital ICs? Give example of them.

- Expand A+B to minterm and maxterms.

- Draw a 4-variable K-map.

- Subtract 11000 from 00100 using 2’s complement method.

- What is race around condition of a JK flip flop? How can it be eliminated?

- Design a circuit that gives an output AB+AB using discrete electronic circuits.        (4)

- Answer any one (01) 									     (5)

- Realized the AND gate with the help of Resistor-Transistor Logic (RTL). Explain briefly with the help of the circuit diagram.

- Draw the logic diagram of a master- slave JK flip flop and explain its operation with the help of a truth table.